[BAND NAME]  TECH SPEC

Version 1.0  ·  [DATE]  ·  On-the-night contact: [NAME] [PHONE]

Template by Seismic DSP · seismicdsp.com/guides/tech-specs. Replace everything in [brackets], delete anything that doesn't apply (it's deliberately complete), and delete all red text like this before sending.

About the Band

[Big Doinks is a four-piece guitar focussed down-tempo doom band. Drums (SPD pad running 2 backing tracks), bass (amp on stage), 2 guitars DI'd from pedalboards, and lead vocals, with backing vocals from guitar and bass.]

Contacts

| ROLE | NAME | PHONE | EMAIL |
| --- | --- | --- | --- |
| On-the-night contact | [NAME] | [PHONE] | [EMAIL] |
| Manager | [NAME] | [PHONE] | [EMAIL] |
| Technical | [NAME] | [PHONE] | [EMAIL] |

Stage Plot

[Draw or paste your stage plot here. Simple shapes and labels: player positions, amps, pedalboards, racks, vocal mic positions, where you hand tails to the venue, and power drops with socket counts.]

Kit List

| GUITAR | BASS | DRUMS |
| --- | --- | --- |
| Pedalboard (DI x 2) | Pedalboard (DI) | 5 piece drum kit + SPD (tracks) |
| [Amp / cab if used] | [Amp / cab if used] | [Breakables needed from venue] |
| Power: 2x 240V, DSR | Power: 2x 240V, DSL | Power: 3x 240V, USC |

Monitors

[We carry our own IEM rack and mixer. We hand you tails from our analog splits.]

[We need one mono drum mix back from FOH on one XLR.]

[We'd like one wedge each side of the stage with a general band mix, no click.]

[We need N monitor mixes from the house console: ...]

Click is NOT routed to the main mix under any circumstances.

| MIX | UNIT | PURPOSE | BAND | FREQUENCY | NOTES |
| --- | --- | --- | --- | --- | --- |
| Mix 1 | IEM TX 1 | Band mix (guitar + bass packs) | 823-832MHz | 823.400 MHz | Licence-free UK/EU |
| Mix 2 | IEM TX 2 | Vox mix | 823-832MHz | 826.300 MHz | Licence-free UK/EU |
| Mix 3 | Drum pack | Drum mix | Wired | — | No RF |
| [Mix 4] | [Unit] | [Purpose] | [Band] | [Frequency] | [Notes] |

Every transmitter you bring, with its exact frequency. Update whenever anything changes; check band legality for every country you play.

Tech Rider

This section scales with the show. At club level, delete most or all of it.

[PA: full-range system capable of 120dBA, clean and undistorted, at the mix position. Subs required.]

[We bring our own FOH engineer. House console is fine: current-generation digital preferred.]

[We bring our own monitor engineer and console at side-stage; we need power and space for it.]

[We bring our own vocal mics; everything else per the input list.]

[Side-stage: one table for the playback rig, 2x 240V, and space for a guitar tech.]

Notes for the Engineer

Guidance, not requirements: how to mix us to OUR taste. FX first, balance second.

[Lead vocal: long dotted-8th delay, big dark hall reverb.]

[Backing vocals: short plate reverb, texture behind the lead, not a second lead.]

[Guitars/synths: FX come off our rig, nothing needed at FOH. Synths sit under the guitars.]

[Drums: big and crushed, kick and snare forward.]

[Screams should cut; clean vocals can sit back.]

Before You Send This

☐  Deleted everything that doesn't apply (including all red text)

☐  Date and version updated, date in the filename

☐  One on-the-night contact named with a working phone number

☐  Stage plot matches the input list and kit list; RF table matches the rack

☐  Two printed copies on you at all times; assume the engineer has never seen your email

Input List

| CH | SOURCE | MIC / DI | LOC | NOTES |
| --- | --- | --- | --- | --- |
| 1 | Kick In | Beta 91 | USC |  |
| 2 | Kick Out | Audix D6 | USC |  |
| 3 | Snare Top | Audix i5 | USC |  |
| 4 | Snare Bottom | SM57 | USC |  |
| 5 | Hi-hat | sE Electronics SE8 | USC | +48V |
| 6 | Rack Tom | Sennheiser e904 | USC |  |
| 7 | Floor Tom | Sennheiser e904 | USC |  |
| 8 | Spare |  |  |  |
| 9 | Overhead L | sE Electronics SE8 | USC | +48V |
| 10 | Overhead R | sE Electronics SE8 | USC | +48V |
| 11 | Bass DI | DI Box | DSL |  |
| 12 | Bass Mic | Audix D6 | CS |  |
| 13 | Guitar 1 | DI (Quad Cortex) | DSR |  |
| 14 | Guitar 2 | DI (Kemper) | DSR |  |
| 15 | BV 1 | Sennheiser e945 | DSR |  |
| 16 | Vox | Sennheiser e945 | DSC | Supplied by artist |
| 17 | Vox Spare | Sennheiser e945 | DSC |  |
| 18 | BV 2 | Sennheiser e945 | DSL |  |
| 19 | Track 1 (Guitars) | DI Box | USC |  |
| 20 | Track 2 (Synth/Vox/FX) | DI Box | USC |  |
| 21 | Track 3 (Sub Bass) | DI Box | USC |  |
| 22 | Click | DI Box | USC | NOT ROUTED TO MAIN MIX. |
| 23 | Drums Shout | Switched SM58 | USC | NOT ROUTED TO MAIN MIX. Supplied by crew |
| 24 | Sidestage Shout | Switched SM58 | SR | NOT ROUTED TO MAIN MIX. Supplied by crew |

* Mics listed are a preference. Where the listed make/model is unavailable, an industry-standard equivalent is acceptable.